CENG4120 Report

Report on Clock Tree Synthesis

Group Team 08

Wong Wai Chun 1155173231

Yip Ching Wai 1155145400

Overview

This Python script provides a comprehensive solution for Clock Tree Synthesis (CTS), which is a crucial step in the design of circuits to ensure that the clock signal reaches all elements of the system in a synchronized fashion. It reads configuration parameters and node information from an input file, processes this data through different routing algorithms, and outputs the results to a specified file.

Script Structure and Modules

Argparse Module: Used for parsing command-line options, enabling the script to accept inputs and outputs from the user.

Matplotlib and Numpy: Utilized for plotting the distribution and connections of pins and taps on a grid and for handling numerical operations and optimizations, respectively.

Heapq: Facilitates efficient priority queue management, which is vital for some of the routing algorithms used.

KMeansConstrained: An implementation of the k-means algorithm that adheres to specific constraints, ideal for clustering pins to taps based on specified rules.

Main Components

- Data Structures:

Pin and Tap classes model the essential components of the CTS problem.

Node and Tree classes are used in hierarchical routing to manage the connectivity of pins and taps.

- Input Parsing (parse_input):

Reads and interprets an input file specifying maximum runtime, load constraints, grid size, and the coordinates of pins and taps.

- Grouping methods

Direct Assignment (assign_pins_to_taps): Assigns pins to the nearest available tap based on Manhattan distance, adhering to load constraints.

Problem of Direct Assignment

Since it directly assigns the pin to tap has a huge problem when the tap max load is full, the pin can be assigned far aways from the closer pin and cause unexpected results. So we decided to use a better method to group all the pin and tap.

Constrained Clustering (assign_pins_to_taps2): Utilizes a constrained k-means clustering algorithm to distribute pins optimally among taps, respecting maximum load constraints per tap.

Result:

- Routing Algorithms:

Prim’s Algorithm for MST (prim_mst): Constructs a minimum spanning tree to minimize the total wire length connecting pins and taps.

Recursive Spatial Algorithm (perform_rsa_routing_optimized): Uses a heap-based approach to merge nodes recursively until one tree remains.

Hierarchical Routing (perform_hierarchical_routing_optimized): Another approach where nodes are merged based on their proximity to optimize the routing tree structur

Output and Visualization:

Plotting (plot_assigned_pins_taps and plot_connections): Visual representations of the grid and connections for analysis and verification.

Output Writing (write_output_file): Documents the final connections in a specified output file format.